# Wetland Drainage User Guide

9/10/ 2020

Version 1.1

# Introduction

## Scope and Purpose

The product is called Wetland drainage decision tool. The purpose of the product is to help agricultural producers and other stakeholders understand the land and resource allocation decisions in crop production, wetland hydrology and watershed management in the Canadian Prairies. The spatial resolution is at the Census of Agricultural Regions in the Canadian Prairies (Figure 1).

Figure 1. Census of Agricultural Regions in Canadian Prairies

## Process Overview

In this section, the main components of the wetland drainage decision tool are described. In the descriptions, we categorize input variables into decision and fixed variables. The decision variables are those that can be changed by the user whiles the fixed variables cannot be changed by the user; the fixed variables have already been specified in the system to run it. Also, the assumptions underlying the calculation of the components of the decision tool are stated. Below is a flow of the main components of the system:

Revenue

  - Yield

  - Price

  - Crop Insurance

  - Canadian Agriculture Income Stabilization Program (CAIS)

Cost

- 2.1. Drainage Cost

- 2.1.1. Drainage Construction

- 2.1.2. Drainage Rehabilitation Cost

- 2.1.3. Drainage Maintenance Cost

- 2.1.4. Nuisance Cost

- 2.1.5. Input Wastage Cost

- 2.2. Production Input and Machinery Cost

Time Available for Drainage

Net Present Benefit of Drainage and Decision Rule

- Farm Revenue

The revenue component is the positive cash flow component of the farm enterprise; it brings money to the operator. The main components are crop yields (Y), price of the crops (P), crop insurance, and Canadian Agriculture Income Stabilization Program (CAIS). It is assumed that the planning horizon of the farm is 20-years, which means that in that period the farmer will be receiving revenue from farm operations. Revenue for a farming season is as:

Revenue=  14YiPi +crop insurance+CAIS payments

Crop Yield

We estimate a bio-economic crop production function at the Census of Agricultural Regions scale in the Canadian Prairies. The production function is a modified version of the one used in Cortus (2015), and it is consistent with other crop production functions estimated in North America (Table 1 in appendix 1). The production function for the ith crop is specified in quadratic functional form as:

Yi=f(CAR constants,  lagoutput price, input use level, fertiliser price index,

Square of input use level,  GSP/GDD, square GSP/GDD,  trend term)

Notes: CAR denotes Census of Agricultural Regions; GSP denotes growing season precipitation; GDD denotes growing season growing degree days; the input use level is derived using the equality of the value of marginal product to input price (the derivation is provided in appendix 1).

Assumptions:

- The operator follows a 5-year crop rotation: Canola, Barley, Flax, Spring Wheat and Summer-fallow.

- Growing season is from May 15th to August 13th.

- Period of analysis is from 1990 to 2020.

Decision variables: Census of agriculture region specific-constant (the region of the operator is assigned value of 1 if the user is in a specified CAR, and zero otherwise), and crop price last year.

Fixed variables: All other variables are controlled by the decision tool system. In particular, a logistic distribution is assumed for the climatic variables, to predict values to predict the yield fucntion. All other variables are fixed at the means of the historical data of that variable.

Crop Insurance

The farmer receives compensation if the yield of quarter-section falls below the insurance coverage level. The default coverage level is 70% of the yield of the crop determined by the insurance program.

Decision variable: The first variable is a dummy variable which is 1 if the user is in an insurance program. The other variable is coverage level, which the user will change depending on the specifics of his/her insurance program.

Canadian Agriculture Income Stabilization Program

The program offers protection to farmers that experience negative deviations from their average farm production returns.

Assumptions:

- Three tiers of protection levels; 0-70% (tier1), >70- 85% (tier 2) and >85 – 100% (tier 3). If farmer’s production return is below the protection level of the average level of return, a payment to the farmer is triggered.

- Average level of return is an Olympic 5-year average margin for crop production income and expenses (excluding drainage related cost).

- Payment to the farmer is a shared between the farmer and government (farmer: government) and it is based on the given protection level (20%: 80% (tier 3), 30%:70% (tier 2) and 50%: 50% (tier 1)).

- Payments are zero for negative returns.

- Farmers’ deposits to the program are captured as cost.

Decision variable:

- Dummy variable denoting 1 if the farmer decides to join the program and zero otherwise.

- Protection level.

- Cost

The cost component is a negative cashflow to the farm (it takes money or resources away from the farm). The main components are crop production input cost (cpc), machinery cost (mc), CAIS deposits (cd), and drainage related cost, including drainage construction cost (dc), nuisance cost (nc), input wastage cost (iwc).

Cost=  cpc+mc+dc+nc+iwc+cd

Input costs

They are the costs associated with production inputs that are used to produce the crops. The costs are taken from representative farm budgets in the Canadian Prairies.

Assumptions:

- The inputs costs are seed, pest control, and fertilizing.

- Input cost varies by the soil zone and tillage practice

- Farmer applies all pesticides himself except fungicide which is applied by high clearance sprayer because fungicides are applied at later crop production stage.

- Default levels of input use and unit prices are taken from representative farm enterprise budgets from the Canadian Prairies.

Decision Variables:

- Dummy variables (1 or 0) for seed, pest control and fertilizer are specified. A producer chooses 1 for each of the variables if he/she applies that input on the farm, and zero otherwise.

- The soil zone of the producer.

Machinery Costs

Machinery cost comprise fuel and repair, and actual machine use cost, including the cost of non-harvest, harvest, and summer fallow operations. Machine types are grouped into powered equipment and drawn equipment. In the powered machinery group, we have tractors (differentiated by their horsepower), and combines (differentiated by classes: 5 to 7+); in the drawn group we have seeders, sprayers, and swathers which are all differentiated by their width. The default values of annual hours of use, work rate (ha/hour) and operating cost (hour) of machinery use are taken from representative machinery custom and rental rate guide in the Canadian Prairies.

Assumption: Producers practice no tillage farming.

Decision variables:

- Farmers species a value of a dummy variable, which is 1if he/she uses any of the machines specified above, and zero otherwise

- If a machine is used, is annual hours of use, work rate (ha/hour) and operating cost (hour) are specified.

Drainage Cost

The cost of wetland drainage comprises ditch construction, drainage rehabilitation, and drainage maintenance.

- Drainage Construction Cost

This is the cost of constructing wetland drainage.

Assumption:

- Contour surface drainage

- Complete drainage of all wetlands in a quarter-section

The formula for wetland drainage cost is given as:

DC =  dl*cdl+imc

Notes: dl is ditch length; cdl is the number of hours needed to complete a meter of ditch; imc is machinery cost of scrapper ($/hour).

Decision variable:

- The farmer chooses the levels of the variables above.

- Rehabilitation Cost

Cost incurred for preparing drained wetland areas for seeding. Rehabilitation cost of $260/ha is used for this study ($200/ha in Cortus (2015) converted to 2020$).

- Maintenance Cost

It is 1.5% of wetland drainage cost (Cortus, 2015)

Assumption:

- Producers use 1.5% of the time spent in constructing ditches to conduct maintenance.

- Since drainage are maintained, its usefulness horizon could be infinite.

Nuisance cost

They are the cost that result from wetland obstruction to machinery use on a quarter-section. Nuisance factors are calculated from the relationship between number of wetlands and farm size (Cortus, 2015). Then, nuisance cost is calculated as nuisance factor multiplied by machinery operating cost.

Input Wastage Cost

The presence of wetlands will also result in input waste from This cost is positively related to the additional time of machinery spent on the field and the area overlapped with inputs (10% is assumed for this study).  It is calculated as the multiplication of nuisance factor by 0.1 by machinery operating cost. The factor 0.1 is 10% overlap.

- Time Available for Drainage

The time (hours) available for drainage is the residual time after the time for all other operations (crop harvesting, existing drainage maintenance, fall weed spraying, summer fallow operations) are taken from the total working days during the period August 1st to Nov 11th.

Assumptions:

- Workdays are from ripe to freeze-up (August 1st to Nov 11th)

- Workdays and non-workdays are calculated from workday and non-workday conditional and unconditional probabilities (Cortus 2015).

- Producers will take at most 3 years to complete all drainage.

- When drainage is started it will not stop until completed.

Harvest

The start date of harvest is based on crop maturity date which is also determined by the growing degree days. Different crop maturity dates given GDD for different crops are provided in Cortus (2015). The number of days for harvesting will be the sum of the number of days for swathing and combining.

Other Farm Operations

The total hours for fall weed spraying, maintaining existing drainage infrastructure, and summer fallow operations are calculated.

Decision variables: The total hours for each of the above farm operations.

- Wetland Drainage Decision Criteria

The difference between the total revenue and the total cost is net-returns for a quarter-section. Also, the difference between the net-returns from quarter-sections with drained wetlands and quarter-sections with wetlands (not drained) is the net-returns from wetland drainage. The net-present benefit of wetland drainage (NPB) is the present value of net-return from wetland drainage over 20-year period, given a discount rate.

Moreover, the expected total time required for drainage in hours is the total number of hours required to drain wetlands on the quarter-section. The total number of hours is the ditch length for draining the wetlands multiplied by the number of hours for constructing a meter of the ditch.

Decision Rule

- First Question: Is the total time available for wetland drainage in a quarter-section less than or equal to the total time required for constructing wetland drainage ditches? If no, the producer will not even consider wetland drainage, and might consider re-evaluating question (1) in the next growing season. If yes, then the producer will consider the second question below.

- Second question: Is the calculated NPB positive? If yes, the farmer will drain the wetland.

Summary: The producer will drain the wetland on quarter sections if the answer to question 1 is yes and NPB is positive.

Appendix 1

Table 1. Empirical Crop Production Functions in North America

| Paper | Journal | Studied Crops | Time Period | Region | Estimation Method & Functional Form | Weather | Price | Technology | Land Quality & Characteristics |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Qui et al (2015) | Nature | Corn, Soybeans | 1950-2013 | Ontario, Canada | Ordinary Least Squares (OLS) Method; Quadratic functional form | Precipitation before GS, precipitation after GS, GDD | Lag Output price, Fertilizer price index | Trend term |  |
| Houck & Gallagher (1976) | American Journal of Ag. Econs | Corn | 1951-1971 | US | OLS method, Quadratic functional form for weather | Weather: proxy variable by USDA to capture moisture conditions for pasture in selected states during growing season | Ratio of fertilizer price index to price of corn | Trend term |  |
| Kaufmann & Snell (1997) | American Journal of Ag. Econs | Corn | 1889-1995 | Ontario, Canada | OLS method Log-linear functional, form Quadratic functional form | Rainfall and temperature | Lag of corn prices, and change in input (derived from value of marginal product = input price) | Time trend, real value of machinery per acre | % change in acres planted from one observation to another |
| Schlenker & Roberts (2009) | PNAS | Corn | 1950-2005 | US | Nonparametric method Step functional form for heat 8th degree Polynomial for heat Piecewise linear functional form for heat Quadratic functional form for time trend | Temperature withing growing season, precipitation | None | Quadratic time trend | None |

Table 1 Continued. Empirical Crop Production Functions in North America

| Paper | Journal | Studied Crops | Time Period | Region | Estimation Method & Functional Form | Weather | Price | Technology | Land Quality & Characteristics |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cabas et al. (2010) | Climate Change | Corn, Soybeans | 1981-2006 | Ontario, Canada | Feasible Generalize Least Square Method Just and Pope production function | Length of growing season (GS), GS temperature, GS precipitation, coefficient of variation of GS precipitation, coefficient of variation of GS temperature; and quadratic terms of weather variables | change in input (derived from value of marginal product = input price) | trend | Change in cultivated area |
| Tolhurst & Alan (2015) | American Journal of Ag. Econs | Corn, Soybeans | 1955-2011 | US | Nonparametric method, Quadratic functional form for precipitation | Precipitation, growing degree days, extreme temperature degree days, vapor pressure deficit |  | trend |  |
| Miao et al. (2016) | American Journal of Ag. Econs | Corn, Soybeans | 1977-2007 | US | Instrumental Variable method, Quadratic functional form | Degree days, overheat days, temperature deviations, | Output price, lag fertilizer price index, Federal Ag. Improvement Act dummy | Trend |  |
| Cortus et al (2015) | Canadian Journal of Ag Econs | Canola, Barley, Flax, Spring Wheat |  | Saskatchewan | Systems of equations, quadratic functional form | GS precipitation, GDD |  |  |  |
| Jiangui et al (2020) | IEEE Journal of Selected Topics in Applied Earth Observations And Remote Sensing | barley, canola, and spring wheat | 2000 - 2016 | Canadian Prairies (Census of Agriculture Regions) | Ordinary least squares, linear functional form | MODIS metrics |  | Trend term |  |

Derivation of Input Level Variable

Profit maximization posits that the value of marginal product of an input must be equal price of the input:

VMPx = Py MPx = Px

Py ∆Y∆Qx= Px   .  Input ∆ = ∆Qx = Qx,t- Qx,t-1= (Py (Yt-Yt-1))/ Px,t

Py:  Crop output price

Px,t  index of crop input prices paid by farmers

Yt & Yt-1: Crop yield in time t and lag of crop yield, respectively.